ICSE Guess Paper - 2008

Class - X

Subject - Physics

CH. – 8 CURRENT ELECTRICITY

Question – 1. Mention two factors on which the resistance of a wire depends.

Question – 2. Mention two factors on which the internal resistance of a cell depends.

Question – 3. State Ohm’s law. Write an expression for the resistance of a conducting wire in terms of its length and area of cross section.

Question – 4. Distinguish between potential difference and emf of a cell.

Question – 5. Derive an expression for equivalent resistance when two resistors of resistance R1 and R2 are connected in (i) parallel and (ii) series.

NUMERICALS

Numerical – 1. A cell supplies a current of 0.6 A through a 2 ohm coil and a current of 0.3 A through a 8 ohm coil. Calculate the emf and the internal resistance of the cell.

Solution :- Here, I1 = 0.6 A, R1 = 2 Ω, I2 = 0.3 A, R2 = 8 Ω, r = ?

Using, I = E/(R + r), we get

0.6 = E /(2 + r)   ---   ---   ---   ---   (i)

0.3 = E / (8 + r)   ---   ---   ---   ---   (ii)

from (i) and (ii) we get  0.6(2 + r) = 0.3(8 + r)

or,  0.3 r = 1.2

or,        r = 4 ohm.

Substituting the value of r in equation (i) we get

E = 0.6(2 + 4) 3.6 V

Hence, r = 4 ohm, E = 3.6 V. [Ans.]

Numerical – 2. Four cells each of emf 2 V and internal resistance 0.1 ohm are connected in series.  The combination is connected in series to an ammeter of negligible resistance, a 1.6 ohm resistor and an unknown resistor R1 . The current in the circuit is 2 A.  Draw a labelled circuit diagram for the above arrangement and calculate :

(i) the total resistance of the circuit

(ii) the total emf

(iii) the value of R1

(iv) the potential difference across R1.

Solution :- The labelled circuit diagram is given below :

Fig

Total internal resistance = 4 × 0.1 Ω  = 0.4 Ω

(i) Total resistance = 1.6 + 0.4 + R1 = (2 + R1) Ω

(ii) Total emf = (2 + 2 + 2 + 2) V = 8 V

(iii) Using,   I = total emf / total resistance

or,    2 = 8 / (2 + R1)

or,    2R1 + 4 = 8

or,         R1 = 2 Ω

(iv)  Potential difference across R1  = I R1 = 2 × 2 = 4 V

from (i) total resistance = 2 + R1 = 2 + 2 = 4 Ω

Numerical – 3. Four cells each of emf 1.5V and internal resistance 2 ohm are connected in parallel. The battery of cells is connected to an external resistance of 2.5 ohm. Calculate :

(i) the total resistance of the circuit

(ii) the current flowing in the external circuit and

(iii) the drop in potential across the terminal of the cells.

Solution :- The arrangement of the four cells in parallel is as under :

Fig.

(i) As the cells are connected in parallel, the net emf of battery = 1.5 V,

Let the net internal resistance be r

Therefore, 1/r = 1/2 + 1/2 + 1/2 + 1/2 = 2  or, r = 0.5 Ω

(ii) The total resistance R of the circuit is given by

R = 0.5 + 2.5 = 3 Ω

Current flowing in the circuit,  I = V/R = 1.5/ 3 = 0.5 A

(iii) Potential difference across the battery, V = IR = 0.5 ×2.5 = 1.25 V

Numerical – 4. Four resistances of 2.0 Ω each are joined end to end to form a square ABCD. Calculate the equivalent resistance of the combination between any two adjacent corners.

Solution :-    In this situation three resistors are connected in series and their combination in parallel with the fourth resistor as given below :

Fig.

By series combination we get,   2 + 2 + 2 = 6 ohm

This 6 ohm resistor is connected in parallel with fourth 2 ohm resistor. Let the resistance of the combination be R

Therefore, 1/R = 1/6 + 1/2  = 2/3

Hence,                      R =  3/2 ohm.

Numerical – 5. The following circuit diagram shows three resistors 2 ohm, 4 ohm and R ohm connected to a battery of emf 2 V and internal resistance 3 ohm. A main current of 0.25 A flows through the circuit.

Fig
.

(i) What is the pd across the 4 ohm resistor ?

(ii) Calculate the pd across the internal resistance of the cell.

(iii) What is the pd across R and 2 ohm resistors ?

(iv) calculate the value of R.

Solution :-